# ThinManager 2.5 Interface

## Opening ThinManager

ThinManager is used for the configuration of ThinManager Ready Thin Clients in the ACP thin client environment.

ThinManager can be started using any of the traditional Windows methods, based on the administrator's preference:

- Run from the Start Menu, Start>Programs>ACP>ThinManager.

- Run from the Run line, Start>Run> C:\Program Files\Automation Control Products\ThinManager\ThinManager.exe (default path).

- Run from a command prompt, C:\Program Files\Automation Control Products\ThinManager\ThinManager.exe (default path).

- Run from a shortcut on the desktop.

- Run from a ThinManager icon in the system tray, if this option is selected in View>Options from the menu bar.

- Run from Windows Explorer.

Note: ThinManager can be run on a terminal with full privileges if the user is an administrator or a member of the ThinManager Administrator’s group.

## ThinManager Graphic User Interface

ACP ThinManager 2.5 Graphic User Interface

The ThinManager administrative interface provides "at-a-glance management". The groups and terminals are displayed in the tree pane. The configuration data is displayed in the detail panel. Color-coded icons in the tree pane show the on-line status of terminals.

The sections of the ThinManager interface include:

- A Title Bar  with the standard Windows Minimize/Maximize/Close shortcut icons.

- A Menu Bar  with commands.

- A Tree Pane  with an expandable/collapsible tree showing the ThinManager Servers, Terminal Servers, Terminal Server Groups, Groups, and Terminals in the ThinManager network. Terminals that are on-line have a green monitor icon, while stopped or rebooting terminals have a red monitor icon.

- A Details Pane  with information about settings and configurations. The blue group icon denotes a property that was obtained from the group. The details pane is tabbed for organization. The tabs that are shown depend on the tree item that is highlighted.

- A Status Bar  that shows advice and tips.

- The Communication Indicator  shows green when ThinManager is talking to a ThinManager Server. ThinManager will wait until this communication is finished before processing additional requests.

## Tree Pane

The tree pane shows the members of the ACP Thin Client Network in an expandable tree.

ACP ThinManager 2.5 With Tree

The current version of the Tree separates the ThinManager Servers, Terminal Servers, and Terminal Server Groups. Although a single computer can be a ThinManager Server, a Terminal Server, and a member of a Terminal Server Group, as shown in the example, these are three distinct functions that are displayed in the tree to reflect its function.

ThinManager Servers can be expanded to show the Groups and Terminals under them. Groups can be expanded to show the Terminals underneath them. Terminals can be expanded to show the Terminal Servers or Terminal Server Groups they are assigned to. Terminal Server Groups can be expanded to show the member Terminal Servers.

## Icons

### Tree Icons

Several of the menu tools and other features are dependent on what icon is highlighted in the tree.

Three Tree Branches

The tree is divided into three branches, ThinManager Servers, Terminal Servers, and Terminal Server Groups.

- A blue icon of ThinMan represents the ThinManager Servers

- A blue server represents the Terminal Servers

- A pair of blue servers represents the Terminal Server Groups.

Each of the branches of the tree can be expanded.

ThinManager Server Icons

A Blue ThinManager Server icon represents the top level of the tree and can be collapsed or expanded. This branch shows the local ThinManager Server by default. Other ThinManager Servers can be displayed by selecting Edit>Add ThinManager Server from the menu bar. See Add ThinManager Server for details.

A Green ThinManager Server icon represents a ThinManager Server that has an active communication link with the program. These can be collapsed or expanded to show the Group and Terminal icons that nest under the ThinManager Server icons.

A Red ThinManager Server icon represents a ThinManager Server that is not communicating with the program.

Terminal Server Icons

A Blue Server icon represents the top level of the Terminal Server tree and can be collapsed or expanded.

A Gray Server icon represents individual terminal server. These can be collapsed or expanded to show the Terminal icons that nest under the Terminal Server icons.

Terminal Server Group Icons

A Blue Terminal Server Group icon represents the top level of the Terminal Server Group tree and can be collapsed or expanded.

A Gray Terminal Server Group icon represents individual terminal server groups. These can be collapsed or expanded to show the terminal server icons that nest under the Terminal Server Group icons and the terminals assigned to that terminal server.

Group and Terminal Icons

A Group is represented by an icon of two monitors with a blue screen.

A Terminal is represented by an icon of a single monitor.

A Red terminal screen indicates that the Terminal is off or not communicating with the ThinManager Server.

A Green terminal screen indicates that the Terminal is on and communicating with the ThinManager Server.

A Locked terminal or group icon means that that terminal or group is being modified and cannot be changed by another user. If the Group or Terminal remains locked after the Properties is closed, they can be unlocked by using Edit>Unlock from the menu bar.

Each item of this tree level can be expanded to show what Terminal Server Groups or Terminal Servers it is assigned to.

Expanded Tree Showing Icon Variety

The tree in ThinManager example has been expanded to provide greater detail about the status of the terminals.

Group and Terminal Nesting

Each Group can be expanded to show the terminals that are members of the group.

Each Terminal can be expanded to show the Terminal Server Groups or Terminal Servers that it is assigned to.

Each Terminal Server Group can be expanded to show the Terminal Servers that are assigned to it.

Each color represents a status state.

Terminal Server Connection Icons

Under each Terminal are icons representing the Terminal Servers that they are assigned to. The lightning bolt color indicated the connection status.

- A Red lightning bolt represents a lack of connection to the terminal server.

- A Green lightning bolt represents a connection to the terminal server with an active session.

- A Yellow lightning bolt represents a connection to the terminal server with an active session that is the backup in Instant Failover mode.

Terminal Server Nesting

The top level Terminal Server icon can be expanded to show the Terminal Servers that have terminals assigned to them. The Monitor Screen color and the Lightning Bolt color indicate the terminal’s status on the Terminal Server.

Terminal Server Connection Icons

The monitor screen color indicates the ThinManager Server connection status.

- A Red monitor screen indicates that the terminal is off or unable to communicate to the ThinManager Server.

- A Green monitor screen indicates that the terminal is on and able to communicate to the ThinManager Server.

The lightning bolt color indicates the Terminal Server connection status.

- A Red lightning bolt represents a lack of active connection to the terminal server.

- A Green lightning bolt represents a connection to the terminal server with an active session.

- A Yellow lightning bolt represents a connection to the terminal server with a session that is the backup session in Instant Failover mode.

Disabled Terminal Icons

If a Group or Terminal is disabled using the Tools>Disable function, it will be displayed with a red X over the terminal icon. An entire ThinManager Server or an entire Group can be disabled, but the ThinManager Server icon and the Group icon will not show an X, just the terminal icons.

Once a terminal is disabled, a disabling screen will appear on the terminal until the terminal has been enabled.

- A Red terminal screen with a Red X indicates that the Terminal is disabled and is either turned off or rebooted and waiting to be enabled.

- A Green terminal screen with a Red X indicates that the disabling has been applied to terminal that is turned on. The terminal has a disabling screen and is waiting for enabling.

Disabling Screen

If a terminal is active when it is disabled, it will display the Disable Screen with a message indicating the disabled status in the upper left corner.

Disabled Terminal

If a terminal is booted when disabled, the boot process will be halted until the terminal is enabled.

See Disable Terminals for details.

## Details Pane

The Details Pane has been reorganized in ThinManager 2.5 with tabs to sort information. Highlighting a ThinManager Server, Group, Terminal, Terminal Server, or Terminal Server Group in the tree will display a different set of tabs and the corresponding set of information.

Highlighting the Blue ThinManager Server icon will show no details.

Highlighting a ThinManager Server will show:

- Configuration - These are the configuration parameters set in the ThinManager Server Configuration Wizard and include the Event Messaging summary.

- Licenses - This displays the installed licenses, the quantity used and the quantity available.

- Versions - This displays the version numbers of ThinManager, the firmware, and the TermCap database.

ThinManager Server Tabs - Licenses

Highlighting a Group will show:

- Configuration - These are the configuration parameters set in the Group Configuration Wizard and include Terminal Server assignments, video settings, and monitoring settings.

- Modules - This lists the assigned modules and parameters for the Group.

- Properties - This is blank for a Group.

- Shadow – This is blank for a Group

Group Tabs - Modules

Highlighting a Terminal will show:

- Configuration - These are the configuration parameters set in the Terminal Configuration Wizard and include Terminal Server assignments, video settings, and monitoring settings.

- Modules - This lists the assigned modules and parameters for the Terminal.

- Properties - This shows the IP address, firmware version, make and model, CPU, memory usage, Terminal Up Time, CPU load, and BootRom version of the Terminal.

- Shadow – This tab shows what the terminal is displaying on its monitor. Administrators can interact with the session while ThinManager Power Users can view the session, but not control it.

See Shadowing for details.

Terminal Tabs - Shadow

Highlighting the blue Terminal Server icon will show:

- Status - This shows the status of the communication connection between the local ThinManager Server and the defined Terminal Servers and ThinManager Servers. This shows whether the local ThinManager Server is able to retrieve the updated resource information used in determining the SmartSession Server Ranking.

Terminal Servers Tabs - Status

The Connection Status of the ThinManager Server may have different messages:

- OK indicates a good connection.

- WTSAPI32.dll connection failed occurs when the terminal server is off or unreachable.

- User specified does not have permission to connect indicates that the Terminal Server has the wrong login information.
See the Terminal Server List wizard for details.

Highlighting a Terminal Server will show:

- Configuration - This shows the installed Client Communication Protocols and the SmartSession settings for the Terminal Server.

- Users - This tab displays information from the Terminal Services Manager. It shows users that are logged into the Terminal Server. Right clicking on a user will show options that allow the session to be Reset (logged off), Disconnected, or have a message sent to it.

- Sessions - This tab displays information from the Terminal Services Manager. It shows users that are logged into the Terminal Server. Right clicking on a user will show options that allow the session to be Reset (logged off), Disconnected, or have a message sent to it.

- Processes - This tab displays information from the Terminal Services Manager. It shows the processes running on the Terminal Server and can have them sorted by Session ID (users) or Process Name. Right clicking on a process will give the option to kill the process.

Terminal Server Tabs - Processes

Highlighting the blue Terminal Server Group icon will show no details.

Highlighting a Terminal Server Group will show:

- Configuration – This will show the members of the Terminal Server Group and details about SmartSession, Multi-Session, and AppLink.

- Server Rank - SmartSession ranks terminal servers based on the available resources. The server with the lowest number has the lightest load. The server with the highest ranking has the highest load.

Terminal Server Group Tabs – Server Rank

### Changing a Terminal’s Group

To change the Group membership of a terminal, open the Terminal Configuration Wizard or the Terminal Properties by either:

- Highlight the terminal in the ThinManager tree pane by clicking on the terminal name or icon and select Edit>Modify from the ThinManager menu bar.

- Right-click on the terminal icon in the ThinManager tree pane and select Modify.

- Double-click on a terminal icon in the ThinManager tree pane.

This will launch the Terminal Configuration Wizard in Wizard mode or the Terminal Properties in Classic mode. On the Identity tab of the Terminal Properties dialog box, use the Group drop-down box to select the new group that the terminal is to be placed in. This will apply the new group configuration to the terminal. If “None” is selected, the terminal will keep its configuration, but will be removed from the group and is treated as an independent terminal.

Note: The terminal will need to be rebooted for the changes to take effect.

### Modifying a Terminal

Modifying a terminal allows you to reconfigure the terminal or change group settings such as touch screen usage, video resolution, or to assign it to a terminal server.

To modify a terminal open the Terminal Configuration Wizard or the Terminal Properties by either:

- Highlight the terminal in the ThinManager tree pane by clicking on the terminal name or icon and select Edit>Modify from the ThinManager menu bar.

- Right-click on the terminal icon in the ThinManager tree pane and select Modify.

- Double-click on a terminal icon in the ThinManager tree pane.

This will launch the Terminal Configuration Wizard in Wizard mode or the Terminal Properties in Classic mode.

Note: The terminal will need to be rebooted for the changes to take effect.

### Deleting a Terminal

A terminal can be deleted from the ThinManager by:

- Highlighting the terminal in the ThinManager tree pane and selecting Edit>Delete from the ThinManager menu bar

- Right-clicking a terminal icon in the tree pane of ThinManager and selecting Delete Terminal.

### Rebooting a Terminal

A terminal or group can be rebooted by:

- Selecting Tools>Reboot Terminals from the ThinManager menu bar. This will reboot all the terminals on the server if the server is highlighted. If a group is highlighted, all members of the group will be highlighted. If a terminal is highlighted, the terminal is rebooted.

- Right-clicking a group icon in the tree pane of ThinManager and selecting Reboot Terminals. This will reboot all the terminals in the group.

- Right-clicking a terminal icon in the tree pane of ThinManager and selecting Reboot Terminal. This will reboot only the highlighted terminal.

### Renaming a Terminal

Terminals can be renamed by:

- Highlighting the Terminal and selecting Edit>Rename.

- Right-clicking the Terminal and selecting Rename.

Note: Renaming a terminal can be accomplished the same way that files or directories are renamed in Windows Explorer. Single click twice on the terminal name; this will draw a box around the name and highlight it. Type the new terminal name.

Renaming a Terminal

The terminal name should be less than 15 characters because of limitations on the terminal server.

Tip on single clicking  twice: Click once to highlight the name, move the mouse slightly and click again. This will prevent Windows from confusing the two single clicks with a double click.